VLSI DPS HW#2

電機四 4109064119 陳柏淳

Q1. LMS filter design

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100101102103104105106107108109110111 | clear all close all clc %% parameters setting Iters = 200; % Iteration Times M = 15; % Filter Size M_end = M + Iters; % Final index W = zeros(M,1); % Filter Coefficients W_hist = zeros(M, M_end); % History of Filter Coefficients mu = 1e-2; % Step size n = 1 : M_end; % Time index s = sin(2*pi*n/16) + cos(2*pi*n/4); % Input d = sin(2*pi*n/16); % Desired e = zeros(M_end); % Error r = zeros(M_end); % RMS rms_step = 16; % Converged Condition converge_value = 0.05/sqrt(2); converged = false; %% LMS Iteration for i = M : M_end U = s(i:-1: i-M+1)'; % Extract input d_hat = W' * U; % Compute filter output e(i) = d(i) - d_hat; % Compute error W = W + mu*e(i)*U; % Update filter coefficients W_hist(:,i+1) = W; % Add coefficients to history r(i) = sqrt(mean(e(max(M, i-rms_step+1):i).^2)); % RMS % Determine convergence if r(i) < converge_value && ~converged disp(['Convergence achieved at ' num2str(i-M+1) ' iterations.']); converged = true; end end disp(['Min RMS value ' num2str(min(r(M:Iters)))]); %% Plot % Plot the RMS Error versus time figure('Name','RSM Plot'); plot(M:M_end, r(M:M_end)); xlim([M M_end]); xlabel('n'); ylabel('RMS Error'); title('RSM versus n'); % Plot the filter coefficients versus time figure('Name','Filter Coefficients'); plot(M:M_end, W_hist(:, M:M_end)'); xlim([M M_end]); xlabel('n'); ylabel('Coefficients'); title('Filter Coefficients'); % Compute and plot the frequency response fft_resp = fft(W, 64); f = (0:63); % frequency axis figure('Name','Frequency response'); plot(f, abs(resp)); xlim([0 63]); xlabel(' Sample points(FFT)'); ylabel('Magnitude'); title(' 64?point FFT to the impulse response with low pass filter'); %% Determine the minimum M Iters = 5000; % Maximum Iteration Times M_max = 100; % Maximum Filter Size mu = 1e-2; % Initial Step size rms_step = 16; % Converged Condition converge_value = 0.05/sqrt(2); convergence_flag = false; for M = 1:M_max M_end = M + Iters; % Final index W = zeros(M,1); % Filter Coefficients n = 1 : M_end; % Time index s = sin(2*pi*n/16) + cos(2*pi*n/4); % Input d = sin(2*pi*n/16); % Desired e = zeros(M_end, 1); % Error r = zeros(M_end, 1); % RMS % LMS Iteration for i = M : M_end U = s(i:-1: i-M+1)'; % Extract input d_hat = W' * U; % Compute filter output e(i) = d(i) - d_hat; % Compute error W = W + mu*e(i)*U; % Update filter coefficients r(i) = sqrt(mean(e(max(M, i-rms_step+1):i).^2)); % RMS % Determine convergence if r(i) < converge_value disp(['Converged at M = ' num2str(M) ', Iteration = ' num2str(i-M+1)]); min_M = M; min_iter_count = i - M + 1; convergence_flag = true; break; end end if convergence_flag break; end end disp(['Minimum iterations for M converged : ' num2str(min_M)]); disp(['Iterations required: ' num2str(min_iter_count)]); |
| --- | --- |

- Result

a. (μ=10-2, Iter=200)

Converged at 95 iterations

Min RMS = 0.0017354

b. The minimum filter length m so that the adaptation can converge in no more than 5000 iterations.

b1. m = 4 when the step size = 10-2

b2. m = 6 when the step size = 10-3

b3. m = 8 when the step size = 10-4

C. Plot

Apply a 64‐point FFT to the impulse response of the converged filter and verify the filter isindeed a low pass one.

Show the plot of filter coefficients bi(n), for i = 0 ~ m-1, versus “n”.

The values of filter coefficients remain mostly unchanged after convergence.

Show the plot of “r(n)” versus “n” and indicate when the filter converges.

95 training samples are required.

Q2. Discrete Wavelet Transform

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100101102103104105106107108109110111112113114115116117118119120121122123124125126127128129130131132133134135136137138139140141142143144145146147148149150151152153154155156157158159160161162163164165166167168169170171172173174175176177178179180181182183184185186187188189190191192193194195196197198199200201202203 | clear all close all clc %% Define filter h_coe = [ 0.852698679009; 0.377402855613; -0.110624404418; -0.023849465020; 0.037828455507]; g_coe = [ -0.788485616406; 0.418092273222; 0.040689417609; -0.064538882629]; q_coe = [ 0.788485616406; 0.418092273222; -0.040689417609; -0.064538882629]; p_coe = [ -0.852698679009; 0.377402855613; 0.110624404418; -0.023849465020; -0.037828455507]; % Symmetric Extension h = Symmetric_Extension(h_coe); g = Symmetric_Extension(g_coe); q = Symmetric_Extension(q_coe); p = Symmetric_Extension(p_coe); % Read img ori_img = double(imread('HW2 test image.bmp')); %% DWT Level 1 [L_1, H_1] = DWT_ROW(ori_img, h, g); [LL_1, LH_1] = DWT_COL(L_1, h, g); [HL_1, HH_1] = DWT_COL(H_1, h, g); % DWT Level 2 [L_2, H_2] = DWT_ROW(LL_1, h, g); [LL_2, LH_2] = DWT_COL(L_2, h, g); [HL_2, HH_2] = DWT_COL(H_2, h, g); % DWT Level 3 [L_3, H_3] = DWT_ROW(LL_2, h, g); [LL_3, LH_3] = DWT_COL(L_3, h, g); [HL_3, HH_3] = DWT_COL(H_3, h, g); % Combine DWT_result = [[[LL_3 HL_3; LH_3 HH_3] HL_2; LH_2 HH_2] HL_1; LH_1 HH_1]; %% Reconstruction % IDWT Level-3 Rec_L_3 = IDWT_COL(LL_3, LH_3, q, p); Rec_H_3 = IDWT_COL(HL_3, HH_3, q, p); Rec_LL_2 = IDWT_ROW(Rec_L_3, Rec_H_3, q, p); % IDWT Level-2 Rec_L_2 = IDWT_COL(Rec_LL_2, LH_2, q, p); Rec_H_2 = IDWT_COL(HL_2, HH_2, q, p); Rec_LL_1 = IDWT_ROW(Rec_L_2, Rec_H_2, q, p); % IDWT Level-1 Rec_L_1 = IDWT_COL(Rec_LL_1, LH_1, q, p); Rec_H_1 = IDWT_COL(HL_1, HH_1, q, p); Rec_img_a = IDWT_ROW(Rec_L_1, Rec_H_1, q, p); % IDWT Level-1 (setting HL1 LH1 HH1 to zero) Rec_L_1_ = IDWT_COL(Rec_LL_1, zeros(size(LH_1)), q, p); Rec_H_1_ = IDWT_COL(zeros(size(HL_1)), zeros(size(HH_1)), q, p); Rec_img_b = IDWT_ROW(Rec_L_1_, Rec_H_1_, q, p); % Calaulate PSNR PSNR_a = PSNR(ori_img, Rec_img_a, 8); PSNR_b = PSNR(ori_img, Rec_img_b, 8); disp(['PSNR a: ',num2str(PSNR_a) ,' dB']); disp(['PSNR b: ',num2str(PSNR_b) ,' dB']); %% Plot % Original image figure('Name','Original Image'); imshow(mat2gray(ori_img)); title('Original Image'); % 3-level DWT figure('Name','3-Level DWT'); Plot_DWT = mat2gray(DWT_result); % add lines num_levels = 3; for level = 1:num_levels region_size = 512 / (2^(level-1)); Plot_DWT(1:region_size, round(region_size/2)) = 1; % Vertical line Plot_DWT(round(region_size/2), 1:region_size) = 1; % Horizontal line end imshow(Plot_DWT); title('3-Level DWT'); % Synthesis A image figure('Name','Synthesis Image A'); imshow(mat2gray(Rec_img_a)); title('Synthesis Image A'); % Synthesis B image(se HL1 LH1 HH1 to zero) figure('Name','Synthesis Image B'); imshow(mat2gray(Rec_img_b)); title('Synthesis Image B'); %% Function % Symmetric Extension function extended_data = Symmetric_Extension(data) Recersed_data = flipud(data); trimmed_data = data(2:end); extended_data = [Recersed_data; trimmed_data]; end % Filter function y = Filter(x, w) if iscolumn(x), x = x'; end if iscolumn(w), w = w'; end N = size(x, 2); M = size(w, 2); L = fix( M/ 2); temp = conv(w, [x(L+1:-1:2), x, x(N-1:-1:N-L)]); y = temp(M : M+N-1); end % ROW-wise DWT function [L, H] = DWT_ROW(img, L_Filter, H_Filter) [row, col] = size(img); L = zeros(row, col); H = zeros(row, col); for i = 1: row L(i,:) = Filter(img(i,:), L_Filter); H(i,:) = Filter(img(i,:), H_Filter); end % Down Sample L = L(:, 1:2:end); % Keep Odd H = H(:, 2:2:end); % Keep Even end % COL-wise DWT function [L, H] = DWT_COL(img, L_Filter, H_Filter) [row, col] = size(img); L = zeros(row, col); H = zeros(row, col); for i = 1: col L(:,i) = Filter(img(:,i), L_Filter)'; H(:,i) = Filter(img(:,i), H_Filter)'; end % Down Sample L = L(1:2:end, :); % Keep Odd H = H(2:2:end, :); % Keep Even end % ROW-wise IDWT function img = IDWT_ROW(L, H, L_Filter, H_Filter) [row, col] = size([L H]); % up sample Ext_L = zeros(row, col); Ext_H = zeros(row, col); Ext_L(:, 1:2:end) = L; % keep odd Ext_H(:, 2:2:end) = H; % keep even for i = 1: row Ext_L(i,:) = Filter(Ext_L(i,:), L_Filter); Ext_H(i,:) = Filter(Ext_H(i,:), H_Filter); end img = Ext_L + Ext_H; end % COL-wise IDWT function img = IDWT_COL(L, H, L_Filter, H_Filter) [row, col] = size([L; H]); % Up Sample Ext_L = zeros(row, col); Ext_H = zeros(row, col); Ext_L(1:2:end, :) = L; % Keep Odd Ext_H(2:2:end, :) = H; % Keep Even for i = 1: col Ext_L(:,i) = Filter(Ext_L(:,i), L_Filter)'; Ext_H(:,i) = Filter(Ext_H(:,i), H_Filter)'; end img = Ext_L + Ext_H; end % PSNR function DWT_result = PSNR(ori_img, Rec_img, nbit) MSE = mean((Rec_img(:) - ori_img(:)).^2); MAXI = 2^nbit - 1; DWT_result = 10 * log10((MAXI^2) / MSE); end |
| --- | --- |

- Result

3-Level DWT Image

Original Image

IDWT synthesis image(set HL1, LH1, HH1 to 0 )

IDWT synthesis image

Synthesis result (PSNR)

a) PSNR = 234.2033 dB

b) PSNR = 23.2903 dB